Notes in Gas Dynamics

G. Kapp

1.1  Gas Equation of State.

The ideal gas law is given by:  PV=nRT

where:

P is the absolute pressure of the gas,

V is the volume of the container holding the gas molecules,

n is the number of moles of gas molecules,

R is the universal gas constant {8.314 J/(mol K) , .08205 (liter atm)/(mol K), etc},

T is the absolute Kelvin temperature.

It must be remembered that the ideal gas law is an EQUILIBRIUM law relating the above quantities.   As such, the statistical quantities pressure and temperature must be well defined in the volume region of concern.

On occasion, it is useful to introduce the gas law using the constants of density and molecular weight (mass).  This is easily done with a few substitutions as follows:

PV=nRTP=nVRTP=ρmwRT                                           Eq. 1.1-1

This can be verified by unit analysis of the ratio n/V .  In SI units,

moles/m3=  kg/m3 * moles/kg  = density * 1/mw

1.2  Acoustic Velocity.

The acoustic velocity of an infinitesimally small pressure wave (speed of sound) is given by:

c2=dPdρ                                              Eq.1.2-1

We wish to derive an alternative form for the acoustic velocity in a gas.

Working with the density,

ρ=mV=mV−1  differentiating,dρ=−mV−2dVdρdV=−mV2                       Eq. 1.2-2

Working with pressure, we assume a adiabatic reversible compression in a gas.  Thus,

PVγ=const.d(PVγ)=P∗d(Vγ)+Vγ∗dP=P∗γ∗Vγ−1∗dV+Vγ∗dP=0soP∗γ∗Vγ−1∗dV=−Vγ∗dPanddPdV=−γPV    Eq. 1.2-2

These two results may now be combined with the ideal gas law to give,

c2=dPdρ=dPdV∗dVdρ=(−γPV)∗(−V2m)=γmPV=γmnRT=γ1mwRTc=γRTmw   =   γPρ

c2=γPρ                                          Eq.1.2-3

1.3  Equation of Continuity.

The equation of continuity, as derived here, is a RATE statement of the law of conservation of mass.

M2

P2



v2

Material initial

M1

P1



v1

Material final

A2                                              A1

Since the process is mass conservative, M1 = M2.  Also take note that the equation as derived, is based on a reference system at rest.  The velocities are “earth” frame of reference.  Also note that the “system” is the chunk of material, not the tube that the material passes through.

M1=M2dm1dt=dm2dtsincedmdt=dmdVol∗A∗dxdt=ρ∗A∗vρ1∗A1∗v1=ρ2∗A2∗v2for the special case of A1=A2,

ρ1v1=ρ2v2                                 Eq. 1.3-1

1.4  Dynamic Equation, the flow F=ma.

This equation may be derived in a number of ways.  Here,  we start with Newton’s Law applied to a control volume capable of being influenced by an external force collection, being accelerated, and non mass conservative.  In this sense, we think of mass as being able to flow both into (+) and out of (-) the control volume.

Fex=d(mv)dt   using the product rule,Fex=m∗dvdt+vr∗dmdt                         Eq1.4-1

where,

m 		is the instantaneous  mass of the control volume,

dv/dt 	is the acceleration of the control volume,

dm/dt 	is the rate of mass gain(+) or loss(-) of the control volume,

Vr 	is the velocity of the control volume, measured from the frame of reference of the inflowing mass, using the +/- of the vector coordinate system.

In the case of multiple matter flow streams (n) into and out of the control volume,  it is useful to invent a flow vector component for each flow stream:

Fex=mdvdt+i=1nvrdmdti                           Eq. 1.4-2

We will now apply the above result to a chunk of material passing through a tube of constant cross section area.   The control volume is the interior of the tube (the material), not the tube.

We will set the constraints as follows:

M1=M2

A1=A2

dv/dt of control volume is zero.

All velocities are measured in earth frame of reference.

The velocity of the control volume (tube interior) is zero.

Control Volume

A2

A1

F2

F1

v2

v1

P2





P1





+

Fex=mdvdt+[vrdmdt]i becomes,

F2−F1=0+(−v2)dm2dt+(−v1)dm1dt                    Eq. 1.4-3

(velocity of control volume, measured from flow stream, is to the left, -)

From Continuity,  dm/dt =  A v   and considered positive if into the control volume,

dm2dt=+ρ2A2v2      and      dm1dt=−(ρ1A1v1)

Substitution gives,

F2−F1=0+(−v2)(ρ2A2v2)+(−v1)(−ρ1A1v1)              Eq. 1.4-4

Using  P=F/A,   F=P A and substitution gives,

P2A2−P1A1=ρ1A1v12−ρ2A2v22

, and with constant area, reduces to

P2−P1=ρ1v12−ρ2v22                                   Eq. 1.4-5

1.5   The Flow Work - Energy Equation.

We start with the standard work energy relationship where the MACRO Work done on (+) the system by an external force is equal to the change in MACRO Kinetic energy of the systems center of mass (KEcm), plus the change in MICRO random energy of the particles which constitute the systems mass, also known as Internal energy of the system (U).

Workex =  KEcmU                             Eq.1.5-1

The internal energy will take into account such items as the materials change in temperature.

We wish to apply the above equation to a material which flows through a tube of constant area.

x2x1

v2

M2

V2

P2

2

U2

v1

M1

V1

P1

1

U1

U2

F2

F1

U1

The process involves pushing a certain quantity of gas ( with speed and internal energy), into, thru, and out of a tube which is fixed in the earth frame of reference.  We fully admit that the gas is compressible although no viscous effects will be considered.  The process will also be considered as adiabatic.  Heat neither entering, nor leaving the system.

We start by considering the external work required.  The work is given by,

Workex=Fex⋅dx

The external work in this process is done by a constant force on an area, this pressure pushing a volume of material into the tube.

Therefore,

Workex=Fex⋅dx=cosΘPexAdx=|Pex||ΔV| cosΘ     where Θ is the angle between the pressure vector  {P,  and the change in volume vector ΔV.

On our system, positive external work (is done by F2,  and negative external work ( is done by F1.

Workex =  P2 V2 – P1 V1  = -PV)                   Eq. 1.5-2

Substituting  the above into the Work – Energy equation we get,

-PV)=  KEcmU

U + PV)}=  KEcm                             Eq. 1.5-3

Note that the quantity enclosed within the braces is the change in enthalpy, H.

Next, we switch our attention briefly to address the change in internal energy, U.  We call on the First Law of Thermodynamics which states that the Heat (+Q) added  to the system, minus the work (+W) done by the system on the surroundings, equals the change in internal energy of the system.

U = Q – W

As can be seen,  with regard to work,  we have a work term in the work – energy equation and also a work term in the first law.  These two equations present a conflict in sign convention.   In order to combine these equations, we will restrict the work term in the first law to expansion of volume of the gas work of the system on its surroundings,  Wegw.  Macro external work in the work – energy equation will remain ‘Workex’ as presented previously.  No shaft work will be considered.  Thus,

U = Q - Wegw                                 Eq. 1.5-4

We now combine the work – Energy equation with the First Law,

Q - Wegw) + PV)}=  KEcm                                Eq. 1.5-5



Our process is adiabatic since no heat will flow through the tube walls, thus Q = 0 so

 0- Wegw) + PV)}=  KEcm

Wegw is the work done by an adiabatic expansion.  It can be shown that this work is given by,

Wegw=P dV=P2V2γV−γ dV    =    Δ(PV)1−γ   (work for the adiabatic process)

We substitute for Wegw  to get,

−−Δ(PV)1−γ+Δ(PV)=ΔKEcm

Δ(PV)1−γ−Δ(PV)=ΔKEcm

(P1V1−P2V2)1−γ−(P1V1−P2V2)=ΔKEcm               Eq.1.5-6

Our system is mass conservative so M1 = M2.  We substitute for cm to obtain,

(P1V1−P2V2)1−γ−(P1V1−P2V2)=12M(v12−v22)

We now factor out   ( P1 V1 – P2 V2)  giving

(P1V1−P2V2)11−γ−1=12M(v12−v22)(P1V1−P2V2)11−γ−1−γ1−γ=12M(v12−v22)(P1V1−P2V2)γ1−γ=12M(v12−v22)(P2V2−P1V1)γγ−1=12M(v12−v22)

We now use  V = M /   to replace the volumes on the left side  giving

(P2Mρ2−P1Mρ1)γγ−1=12M(v12−v22)

(P2ρ2−P1ρ1)γγ−1=12(v12−v22)                                    Eq. 1.5-7

It is important to discuss frame of reference at this point.  Although the total energy of a system is independent of frame of reference, where, and in what form the energy appears, is frame of reference dependent.  For our analysis, KE and thus the velocities are measured in the earth frame of reference.

  - Velocity of Large Amplitude Pressure Wave.

Here we attempt to answer the question: Can a pressure wave move faster than the “speed of sound”?  Assume a gas is enclosed in a cylinder fitted with a piston at one end.  The gas is at rest and at a uniform pressure P1.  The piston now advances at a speed v2 down the tube,  the static pressure near the piston is increased to  P2.    The tube is fixed with respect to the earth and not capable of heat flow through its walls.  All velocities are measured from the earth frame of reference.

The pressure in the far end of the tube does not rise to P2 immediately.  The pressure wave must travel down the tube.  Let us assign the variable “a” to the velocity of this pressure wave.

P2



P1



v1

a

v2

+

Since the state variables are well defined on each side of the wave front, we employ equations:

c12=γP1ρ1                                                        Eq. 1.2-3

ρ1v1=ρ2v2                                                         Eq. 1.3-1

P2−P1=ρ1v12−ρ2v22                                                    Eq. 1.4-5

(P2ρ2−P1ρ1)γγ−1=12(v12−v22)                 	       Eq. 1.5-7

To introduce the pressure wave velocity, we must switch our frame of reference and ride the pressure wave.  This is easily done by noting that in the frame of reference of the pressure wave, the wave’s velocity is zero.  The gas on each side of the pressure wave appears to be moving to the left now as we view it in this frame of reference.  To execute the frame of reference change,  we subtract the speed of the wave, “a”, from all earth frame of reference velocities in each of the three equations above.

The results are:

ρ1(v1−a)=ρ2(v2−a)                       Eq. 1.3-1a

P2−P1=ρ1(v1−a)2−ρ2(v2−a)2              Eq. 1.4-5a

(P2ρ2−P1ρ1)γγ−1=12          Eq. 1.5-7a

Next, we impose the zero earth velocity of the gas on the right side of the piston.  Setting v1= 0, we have:

c12=γP1ρ1                                  Eq. 1.2-3

ρ1(−a)=ρ2(v2−a)                            Eq. 2.1-1

P2−P1=ρ1(−a)2−ρ2(v2−a)2                      Eq. 2.1-2

(P2ρ2−P1ρ1)γγ−1=12                  Eq. 2.1-3

(We should note that the velocity values are still earth frame of reference values)

These equations can be combined to eliminate v2, to give a relationship relating the wave velocity “a”, the acoustic velocity in the still gas “c1”, the pressures of the two regions “P1, P2”, and the ratio of the specific heats “”.

Rearranging Eq. 2.1-1, we get

(v2−a)2=a2ρ12ρ22

This is substituted into both Eq. 2.1-2 and Eq. 2.1-3 to give

P1−P2+ρ1a2ρ2=a2ρ12ρ221ρ2=1a2ρ12(P1−P2+ρ1a2)                               Eq.2.1-4

and

2γ(γ−1)(P2ρ2−P1ρ1)=a2−(P1−P2+ρ1a2ρ2)                   Eq. 2.1-5

Collecting 1/on the right, Eq. 2.1-5 can now be put in the form

2γ(γ−1)P1ρ1+a2=[P1 -P2+ρ1a2+2γP2(γ-1)]1ρ2                Eq. 2.1-6

Equation Eq. 2.1-4 may now be substituted into the above  Eq. 2.1-6 and the resulting equation solved for a2.  The necessary algebra is tedious at best.  It is included for that fact.  Substituting,

2γ(γ−1)P1ρ1+a2=[P1 -P2+ρ1a2+2γP2(γ-1)]P1−P2+ρ1a21a2ρ12

multiply by a212 and expand right side,

2γ(γ−1)P1ρ1a2ρ12+a4ρ12=P12−2P1P2+P22+2P1ρ1a2−2P2ρ1a2+2γP1P2(γ−1)−2γP22(γ−1)+2γρ1a2P2(γ−1)+ρ12a4

remove a412

2γ(γ−1)P1ρ1a2ρ12=P12−2P1P2+P22+2P1ρ1a2−2P2ρ1a2+2γP1P2(γ−1)−2γP22(γ−1)+2γρ1a2P2(γ−1)

collect terms with ‘a’ on the left,

2γ(γ−1)P1ρ1a2−2P1ρ1a2+2P2ρ1a2−2γ(γ−1)P2ρ1a2=P12−2P1P2+P22+2γP1P2(γ−1)−2γP22(γ−1)

factor out 21a2

2ρ1a2γ(γ−1)P1−P1+P2−γ(γ−1)P2=P12−2P1P2+P22+2γP1P2(γ−1)−2γP22(γ−1)

create common denominator using (-1)/(-1) as necessary,

2ρ1a2γ(γ−1)P1−P1(γ−1)(γ−1)+P2(γ−1)(γ−1)−γ(γ−1)P2=(γ−1)(γ−1)[P12−2P1P2+P22]+2γP1P2(γ−1)−2γP22(γ−1)

remove 1/(-1), and other terms from expanding the left side,

2ρ1a2P1−P2=(γ−1)[P12−2P1P2+P22]+2γP1P2−2γP22

factor right side,

2ρ1a2P1−P2=(γ−1)[P1−P2]2+2γP2(P1−P2)

expand left,

2ρ1a2P1−P2=γ[P1−P2]2−[P1−P2]2+2γP2(P1−P2)

divide by (P2-P1)

2ρ1a2=γ[P1−P2]−[P1−P2]+2γP2

2ρ1a2=γP1−γP2−P1+P2+2γP2

Divide by P1. Multiply left side by ,

2γρ1γP1a2=γ−γP2P1−1+P2P1+2γP2P1

a2c12=12γ[γ−1+P2P1(γ+1)]  Since c12 =  P1/ ,

Eq. 2.1-7

It can be seen that when the pressure ratio becomes 1 as in a small amplitude sound wave,  a2 = c12, that is  the wave velocity is the acoustic velocity.  When the pressure ratio increases above 1,  the wave velocity will exceed the acoustic velocity.  This can be seen in the figure 2.2 . Equation 2.1-7 is plotted for a mono, di, and poly atomic ideal gas.

The velocity ‘a’ is the shock wave velocity.  There will be a sharp discontinuity of density and pressure across the wave boundary.   The answer to the question “can a pressure wave move faster than the speed of sound? “ is in fact YES.

Figure 2.2.  Ratio of the wave velocity ‘a’ to the acoustic velocity ‘c1’ as a function of pressure ratio for mono, di, and poly atomic gases.  The figure shows a limiting velocity ratio of 1 when the pressure ratio is 1.  As the pressure ratio increases, so does the velocity ratio.  Pressure ratios less than one are meaningless here.

Equations 2.1-4 and 2.1-5 can also be combined and solved for the density ratio in terms of the pressure ratio and gamma.  Solving Eq. 2.1-4  for a2, we get,

a2=P1−P2ρ1[ρ1ρ2−1]

Solving Eq. 2.1-5  for a2 by multiplying by 2/P1 and regrouping, we get,

2γ(γ−1)(P2P1−ρ2ρ1)+(P1−P2)P1=a2ρ2P1−ρ1P1

We now eliminate a2 by substitution to obtain,

2γ(γ−1)(P2P1−ρ2ρ1)+(P1−P2)P1=(P1−P2)ρ1[ρ1ρ2−1]ρ2P1−ρ1P1

The remaining algebra is again tedious.  It will be outlined as follows.

2γ(γ−1)(P2P1−ρ2ρ1)+1−P2P1=(P1−P2)[ρ1ρ2−1]ρ2ρ1−11P1

2γ(γ−1)P2P1−2γ(γ−1)ρ2ρ1+1−P2P1=(1−P2P1)[ρ1ρ2−1]ρ2ρ1−1

2γ(γ−1)P2P1−2γ(γ−1)ρ2ρ1+1−P2P1=(1−P2P1)[ρ2ρ1−ρ1ρ1][ρ1ρ2−ρ2ρ2]

on the right side, we factor out 1/ , multiply by –1/-1 , to get,

2γ(γ−1)P2P1−2γ(γ−1)ρ2ρ1+1−P2P1=(P2P1−1)ρ2ρ1

density ratio terms are now collected on the right,

2γ(γ−1)P2P1+1−P2P1=ρ2ρ1P2P1−1+2γ(γ−1)

We now solve for the density ratio,

ρ2ρ1=2γP2(γ−1)P1−P2P1+12γ(γ−1)−1+P2P1

and a cosmetic rearrangement of term gives the desired equation:

ρ2ρ1=1+P2P1γ+1γ−1P2P1+γ+1γ−1                                 Eq.2.1-8

Figure 2.3  Density ratio 1 as a function of pressure ratio P2/P1 for mono, di, and poly atomic ideal gasses.  At a pressure ratio of 1, the density ratio is also 1.  As the pressure ratio increases, the density ratio increases to a limiting value of:  4 for mono atomic, 6 for diatomic, and 7 for poly atomic gasses.  Pressure ratios less than one have no meaning.

It should be noted that one could be tempted to compute the density ratio using a combination of the density definition and the function for the adiabatic process trajectory alone, ie. PVconst. and  = M/V.  This would lead to a correct result for a stationary gas experiencing an adiabatic compression, but would be INCORRECT for a gas which is also changing kinetic energy.

Some Numerical Calculations.

We start out with the selection of the di-atomic gas Nitrogen:

N2,  mw=.028 Kg/mole,   = 1.40

The down stream pressure and temperature is P1 = 1 atm = 1.01 x105 n/m2 @ 273 K, while our up stream pressure will be P2 = 2 atm = 2.02 x 105 n/m2 .

First, the down stream gas density is computed using the ideal gas law.

ρ1=P1mwRT=(1.01x105n/m2)(.028Kg/mole)(8.314 j/mole K)(273 K)=1.246  kg/m3

The acoustic velocity down stream is also computed.

c1=γP1ρ1=1.40  (1.01x105 n/m2)1.246 Kg/m3=336.9 m/s

From Eq 2.1-8, the density ratio is computed.

ρ2ρ1=1+P2P1γ+1γ−1P2P1+γ+1γ−1    =1+21.4+11.4-12+1.4+11.4-1=1.625

The shock wave velocity ratio, squared, is computed using Eq 2.1-7

a2c12=12γ[γ−1+P2P1(γ+1)]  =12(1.4)[1.4−1+2(1.4−1)]=1.857

The shock wave velocity is now computed.

a=c11.857=459.1 m/s

The density up stream is computed.

Kg/m3

Finally, the velocity up stream, v2, can be computed from equations Eq. 2.1-1, 2.1-2, or 2.1-3.  All are in agreement.

v2=a1−ρ1ρ2=176.6 m/s       or

v2=a−ρ1a2+P1−P2ρ2         or         v2=a−a2−P2ρ2−P1ρ12γ(γ−1)

will yield the same result.